GATE 2027 PREPARATION LIBRARY

GATE CS — Computer Science & Information Technology

Computer Networks

Layering, switching, data link, routing, IP, TCP and the web

Document CS-CN

First principles → protocols & calculations → GATE-level numericals. Covers the full CS Computer-Networks syllabus with CIDR, routing and TCP.

| Document metadata |  |
| --- | --- |
| Serves | GATE CS (Section 10: Computer Networks) |
| Syllabus basis | Official GATE 2027 (IIT Madras), verified 12 Oct 2026 — see Document 00 |
| Scope | Layering; switching (circuit/packet/virtual) & performance; data link (error detection, MAC, Ethernet); distance-vector & link-state routing; IPv4 (fragmentation, CIDR, NAT); TCP (flow & congestion control, sockets); DNS & HTTP |
| Version | v1.0 — 14 Oct 2026 |

Contents

Chapter 1:  Overview and the Principles of Layering	4

1.1  Why Computer Networks matters for GATE CS	4

1.2  The layered model	4

Chapter 2:  Switching and Performance Metrics	5

2.1  Switching methods	5

2.2  Delay and throughput	5

2.3  Chapter 2 summary	5

Chapter 3:  Data Link Layer	7

3.1  Error detection and correction	7

3.2  Medium access control and Ethernet	7

3.3  Chapter 3 summary	7

Chapter 4:  Network Layer: IPv4 and Routing	8

4.1  IPv4 addressing and CIDR	8

4.2  Fragmentation and NAT	8

4.3  Routing: distance-vector vs link-state	8

4.4  Chapter 4 summary	9

Chapter 5:  Transport Layer: TCP and UDP	10

5.1  TCP vs UDP	10

5.2  Flow control	10

5.3  Congestion control	10

5.4  Chapter 5 summary	11

Chapter 6:  Application Layer: DNS and HTTP	12

6.1  DNS — the Domain Name System	12

6.2  HTTP — the web’s protocol	12

6.3  Chapter 6 summary	12

Chapter 7:  Formula Sheet, Practice Set and Coverage	13

7.1  Formula sheet	13

7.2  A note on previous-year questions (PYQs)	13

7.3  Practice set — instructions	13

7.4  Part A — one-mark questions	13

7.5  Part B — two-mark questions	14

7.6  Answer key and full solutions	15

7.6.1  Part A	15

7.6.2  Part B	15

7.7  Document CS-CN — coverage checklist & verification	15

# Chapter 1:  Overview and the Principles of Layering

## 1.1  Why Computer Networks matters for GATE CS

Networks move data between machines through a stack of cooperating layers. GATE rewards a mix of protocol concepts and calculations (CIDR, delays, TCP windows, routing). This document follows the exact syllabus order.

## 1.2  The layered model

| DEFINITION — Principles of layering Each layer provides services to the one above and uses the one below, hiding its internals behind an interface. A header is added at each layer (encapsulation). OSI (7 layers): Physical, Data Link, Network, Transport, Session, Presentation, Application. TCP/IP (practical): Link, Internet (IP), Transport (TCP/UDP), Application. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Layer |  | Job | Unit | Example |
| Application |  | App protocols | message | HTTP, DNS |
| Transport |  | End-to-end delivery | segment | TCP, UDP |
| Network |  | Routing across networks | packet/datagram | IP |
| Data link |  | Hop-to-hop framing | frame | Ethernet |
| Physical |  | Bits on the wire | bit | — |

Table. Data is encapsulated downward (each layer adds a header) and decapsulated upward at the receiver.

| Done | Chapter | You should be able to… |
| --- | --- | --- |
| ☐ | Ch. 2 Switching & performance | Compare circuit/packet/VC; compute delays & bandwidth-delay product. |
| ☐ | Ch. 3 Data link | Use parity/CRC/Hamming; explain CSMA/CD & Ethernet. |
| ☐ | Ch. 4 Network & routing | Do CIDR/subnetting, fragmentation, NAT; compare DV vs LS routing. |
| ☐ | Ch. 5 Transport | Explain TCP flow & congestion control; sockets; UDP. |
| ☐ | Ch. 6 Application | Describe DNS and HTTP. |
| ☐ | Ch. 7 Formulas & practice | Recall formulas and clear timed numericals. |

# Chapter 2:  Switching and Performance Metrics

How data traverses a network, and the delay/throughput calculations GATE loves.

## 2.1  Switching methods

| Method | How | Note |
| --- | --- | --- |
| Circuit switching | Dedicated path reserved for the whole session | Setup delay; guaranteed bandwidth (telephone) |
| Packet switching (datagram) | Each packet routed independently, store-and-forward | No setup; packets may reorder (the Internet) |
| Virtual circuit | Path set up once; packets follow it in order | Setup + in-order delivery (combines both) |

Table. Datagram packet switching underlies IP; virtual circuits add a connection setup.

## 2.2  Delay and throughput

KEY FORMULA — The delay components Transmission delay = L / R (packet length L bits ÷ link rate R bps). Propagation delay = d / s (distance ÷ signal speed). Total one-hop delay ≈ transmission + propagation (+ processing + queuing). Bandwidth-delay product = R × propagation delay = bits "in flight" on the link.

EXAMPLE — Example 2.1 · Transmission vs propagation delay · Level 3 A 1000-bit packet on a 1 Mbps link (R = 10⁶ bps): transmission delay = 1000/10⁶ = 1 ms. If the link is 200 km and signal speed 2×10⁸ m/s: propagation delay = 200000 / (2×10⁸) = 1 ms. Total ≈ 2 ms.

COMMON MISTAKE / TRAP — Performance traps Transmission delay depends on BANDWIDTH (L/R); propagation delay depends on DISTANCE (d/s) — they are independent. On long fat links the bandwidth-delay product is large, so a big sliding window is needed to keep the pipe full. Store-and-forward adds one transmission delay PER hop.

## 2.3  Chapter 2 summary

KEY TAKEAWAYS — Key takeaways Circuit (reserved path), packet/datagram (independent, store-and-forward), virtual circuit (setup + ordered). Transmission delay = L/R; propagation delay = d/s; bandwidth-delay product = R × prop delay. Store-and-forward costs one transmission delay per hop.

# Chapter 3:  Data Link Layer

The data link layer delivers frames over one hop, detects errors, and coordinates access to a shared medium (Ethernet).

## 3.1  Error detection and correction

| Scheme | Detects / corrects | Idea |
| --- | --- | --- |
| Parity bit | detects odd # of bit errors | One extra bit for even/odd parity |
| Checksum | detects many errors | Sum of words (used in IP/TCP) |
| CRC | detects burst errors | Polynomial division; remainder appended |
| Hamming code | CORRECTS single-bit errors | Parity bits at power-of-2 positions |

Table. CRC is the workhorse for detection; Hamming codes can correct. Hamming distance d detects d−1 and corrects ⌊(d−1)/2⌋ errors.

## 3.2  Medium access control and Ethernet

KEY FORMULA — MAC & Ethernet When many stations share a medium, a MAC protocol arbitrates access. Ethernet uses CSMA/CD: sense the channel, transmit if idle, and if a COLLISION is detected, stop and back off (binary exponential backoff). For collision detection to work, a frame must still be transmitting when the farthest collision returns: minimum frame time ≥ 2 × propagation delay (so Ethernet has a MINIMUM frame size, e.g. 64 bytes). Switched Ethernet (full-duplex) eliminates collisions; hubs share a collision domain, switches separate them.

COMMON MISTAKE / TRAP — Data-link traps A parity bit only DETECTS an odd number of errors; it cannot locate or correct them. CSMA/CD needs the minimum frame size tied to 2× propagation delay — a classic Ethernet numerical. Switches create separate collision domains (one per port); a hub is a single shared domain.

## 3.3  Chapter 3 summary

KEY TAKEAWAYS — Key takeaways Error detection: parity (weak), checksum, CRC (burst errors); Hamming corrects single-bit. Ethernet CSMA/CD: sense, transmit, detect collision, exponential backoff. Minimum frame size ↔ 2 × propagation delay; switches split collision domains.

# Chapter 4:  Network Layer: IPv4 and Routing

The network layer routes packets across networks using IP addresses. CIDR, subnetting, fragmentation and routing are rich GATE territory.

## 4.1  IPv4 addressing and CIDR

KEY FORMULA — Addresses and prefixes An IPv4 address is 32 bits (dotted decimal, e.g. 192.168.1.10). CIDR notation a.b.c.d/n means the first n bits are the NETWORK prefix. A /n block has 2^(32−n) addresses; USABLE hosts = 2^(32−n) − 2 (subtract network and broadcast addresses). Subnet mask for /n has n leading 1s (e.g. /24 = 255.255.255.0, /26 = 255.255.255.192).

EXAMPLE — Example 4.1 · Hosts in a subnet · Level 2 A /26 network: host bits = 32 − 26 = 6 ⇒ 2⁶ = 64 addresses ⇒ 62 usable hosts. Mask = 255.255.255.192. A /30 (common for point-to-point links) has 4 addresses, 2 usable.

## 4.2  Fragmentation and NAT

KEY FORMULA — Fragmentation & NAT If a packet exceeds a link’s MTU, IPv4 fragments it. The fragment OFFSET is measured in 8-byte units; the More-Fragments (MF) flag is 1 on all but the last fragment; DF forbids fragmentation. NAT maps private addresses (10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16) to a public address; PAT also maps ports so many hosts share one public IP.

## 4.3  Routing: distance-vector vs link-state

| Aspect | Distance-vector | Link-state |
| --- | --- | --- |
| Algorithm | Bellman–Ford | Dijkstra |
| What is shared | Distance vectors with NEIGHBOURS | Whole topology, FLOODED to all |
| Knowledge | Only neighbours’ distances | Full network map |
| Problem | Count-to-infinity (slow convergence) | More overhead/computation |
| Example | RIP | OSPF |

Table. DV tells neighbours about the world; LS tells the world about neighbours. LS converges faster but costs more.

COMMON MISTAKE / TRAP — Network-layer traps Usable hosts = 2^(host bits) − 2 (the all-0s network and all-1s broadcast are not assignable). Fragment offset is in 8-BYTE units — divide the byte offset by 8. Distance-vector suffers count-to-infinity; split horizon / poison reverse mitigate it.

## 4.4  Chapter 4 summary

KEY TAKEAWAYS — Key takeaways IPv4 32-bit; /n ⇒ 2^(32−n) addresses, 2^(32−n) − 2 usable hosts; mask has n 1s. Fragmentation: offset in 8-byte units, MF flag; NAT maps private↔public (PAT shares ports). Distance-vector (Bellman–Ford, neighbours, count-to-infinity) vs link-state (Dijkstra, flooded topology).

QUICK REVISION — Can you do these without notes? Compute usable hosts and the mask for a /n block; subnet a network. Compute a fragment offset; explain NAT/PAT. Contrast distance-vector and link-state routing.

# Chapter 5:  Transport Layer: TCP and UDP

The transport layer provides end-to-end delivery between processes. TCP is reliable and controlled; UDP is lightweight.

## 5.1  TCP vs UDP

| Aspect | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented (3-way handshake) | Connectionless |
| Reliability | Reliable (ACKs, retransmission, ordering) | Best-effort (no guarantee) |
| Control | Flow + congestion control | None |
| Overhead / use | Higher; web, file transfer | Lower; DNS, streaming, VoIP |

Table. Processes are addressed by PORT numbers; a socket = (IP, port). The 3-way handshake is SYN, SYN-ACK, ACK.

## 5.2  Flow control

KEY FORMULA — Sliding window (flow control) Flow control stops a fast sender from overwhelming a slow receiver: the receiver advertises a RECEIVE WINDOW (rwnd) = free buffer space, and the sender limits unacknowledged data to it. Throughput ≈ window size / round-trip time (RTT); a window at least the bandwidth-delay product keeps the pipe full.

## 5.3  Congestion control

KEY FORMULA — TCP congestion control TCP keeps a CONGESTION WINDOW (cwnd); it sends min(cwnd, rwnd). Slow start: cwnd starts at 1 MSS and DOUBLES every RTT (exponential) until it reaches the slow-start threshold (ssthresh). Congestion avoidance: above ssthresh, cwnd grows by 1 MSS per RTT (linear — additive increase). On loss: a timeout resets cwnd to 1 and halves ssthresh; three duplicate ACKs trigger fast retransmit/recovery (multiplicative decrease). This is AIMD.

EXAMPLE — Example 5.1 · Slow start · Level 3 Starting cwnd = 1 MSS with ssthresh = 8: cwnd goes 1 → 2 → 4 → 8 over successive RTTs (doubling). At 8 it reaches ssthresh and switches to congestion avoidance (then +1 per RTT: 9, 10, …).

COMMON MISTAKE / TRAP — Transport traps Flow control (rwnd) protects the RECEIVER; congestion control (cwnd) protects the NETWORK — different problems. Slow start is EXPONENTIAL (doubling), not slow in rate of change; the "slow" refers to the small starting window. The sender is limited by min(cwnd, rwnd); throughput ≈ window / RTT.

## 5.4  Chapter 5 summary

KEY TAKEAWAYS — Key takeaways TCP: reliable, connection-oriented (3-way handshake), flow + congestion control; UDP: connectionless, best-effort. Flow control = receiver window (rwnd); congestion control = cwnd with slow start (×2/RTT) then AIMD. Send min(cwnd, rwnd); throughput ≈ window/RTT; socket = (IP, port).

QUICK REVISION — Can you do these without notes? Contrast TCP and UDP and describe the 3-way handshake. Explain flow vs congestion control. Trace cwnd through slow start and congestion avoidance.

# Chapter 6:  Application Layer: DNS and HTTP

The application layer is where user-facing protocols live. The syllabus names two: DNS (naming) and HTTP (the web).

## 6.1  DNS — the Domain Name System

DEFINITION — DNS DNS translates human-readable names (www.example.com) into IP addresses. It is a distributed, hierarchical database: root → top-level domain (.com) → authoritative servers. Resolution can be RECURSIVE (the resolver does all the work) or ITERATIVE (each server refers the client onward). Caching (with TTLs) speeds up repeats. DNS primarily uses UDP (port 53) for queries; TCP for large responses/zone transfers.

## 6.2  HTTP — the web’s protocol

KEY FORMULA — HTTP essentials A client sends a REQUEST (method + URL + headers); the server returns a RESPONSE (status code + headers + body). It runs over TCP (port 80; HTTPS 443). Methods: GET (fetch), POST (submit), PUT, DELETE, HEAD. Status codes: 2xx success, 3xx redirect, 4xx client error (404), 5xx server error. HTTP is STATELESS (each request is independent); cookies add state. HTTP/1.1 persistent connections reuse one TCP connection for multiple requests.

COMMON MISTAKE / TRAP — Application-layer traps DNS is hierarchical and cached; a query may touch root/TLD/authoritative servers, but caching usually short-circuits this. HTTP is stateless by itself — statefulness comes from cookies/sessions, not the protocol. Persistent (keep-alive) connections avoid a new TCP handshake per object, cutting latency.

## 6.3  Chapter 6 summary

KEY TAKEAWAYS — Key takeaways DNS: hierarchical name→IP resolution (recursive/iterative), cached with TTLs, mostly UDP port 53. HTTP: request/response over TCP (port 80), methods GET/POST/…, status 2xx–5xx, stateless (+cookies). Persistent connections reuse one TCP connection for many requests.

# Chapter 7:  Formula Sheet, Practice Set and Coverage

## 7.1  Formula sheet

| Topic | Formula / fact |
| --- | --- |
| Layers | App / Transport / Network / Data link / Physical |
| Transmission delay | L / R (bits ÷ bandwidth) |
| Propagation delay | d / s (distance ÷ speed) |
| Bandwidth-delay product | R × propagation delay |
| CIDR /n | 2^(32−n) addresses; 2^(32−n) − 2 usable hosts |
| Fragment offset | byte offset ÷ 8 |
| Routing | DV = Bellman–Ford (RIP); LS = Dijkstra (OSPF) |
| TCP | 3-way handshake; send min(cwnd, rwnd) |
| Congestion control | slow start (×2/RTT) → AIMD; loss halves/ resets |
| Throughput | ≈ window / RTT |
| DNS / HTTP | DNS UDP/53 (hierarchical); HTTP TCP/80 (stateless) |

Table. One-page Computer-Networks recap (consolidated in the Formula Book, M2).

## 7.2  A note on previous-year questions (PYQs)

COMMON MISTAKE / TRAP — No fabricated PYQs This library never invents a past question, year or official answer. Verified PYQs with sources are in PYQ-CS. Below is ORIGINAL GATE-style practice.

## 7.3  Practice set — instructions

Attempt timed before the solutions. Part A = 1-mark; Part B = 2-mark. GATE marking as usual.

NOTE — Answer key is separated Solutions start on a new page (Section 7.5).

## 7.4  Part A — one-mark questions

Q1  [1M · NAT]  How many USABLE host addresses are in a /26 network?

Q2  [1M · MCQ]  TCP is:  (a) connectionless, unreliable  (b) connection-oriented, reliable  (c) connectionless, reliable  (d) connection-oriented, unreliable.

Q3  [1M · MCQ]  Routing between networks is the job of the ___ layer:  (a) data link  (b) network  (c) transport  (d) application.

Q4  [1M · NAT]  How many 1-bits are in the subnet mask of a /24 network?

Q5  [1M · MCQ]  CSMA/CD is the access method of:  (a) token ring  (b) Ethernet  (c) ATM  (d) PPP.

Q6  [1M · MSQ]  Which are features of TCP? (a) reliability  (b) connection-oriented  (c) flow control  (d) connectionless.

Q7  [1M · NAT]  Transmission delay (ms) for a 1000-bit packet on a 1 Mbps link?

Q8  [1M · MCQ]  DNS queries primarily use:  (a) TCP port 80  (b) UDP port 53  (c) TCP port 25  (d) UDP port 123.

## 7.5  Part B — two-mark questions

Q9  [2M · NAT]  How many usable hosts are in a /30 network?

Q10  [2M · MCQ]  Distance-vector routing is based on:  (a) Dijkstra  (b) Bellman–Ford  (c) spanning tree  (d) flooding only.

Q11  [2M · NAT]  How many total addresses are in a /28 block?

Q12  [2M · MCQ]  Link-state routing uses:  (a) Bellman–Ford  (b) Dijkstra  (c) RIP  (d) distance vectors.

Q13  [2M · NAT]  TCP slow start: cwnd starts at 1 MSS and doubles each RTT. What is cwnd after 3 RTTs (in MSS)?

Q14  [2M · MSQ]  Which are PRIVATE IPv4 ranges? (a) 10.0.0.0/8  (b) 192.168.0.0/16  (c) 8.8.8.0/24  (d) 172.16.0.0/12.

Q15  [2M · NAT]  A fragment starts at byte offset 800. What value goes in the fragment-offset field (8-byte units)?

Q16  [2M · MCQ]  Flow control in TCP primarily protects:  (a) the network  (b) the receiver  (c) the router  (d) the sender’s CPU.

Q17  [2M · NAT]  Propagation delay (ms) for a 200-km link at 2×10⁸ m/s?

Q18  [2M · MCQ]  An HTTP status code of 404 indicates:  (a) success  (b) redirect  (c) client error (not found)  (d) server error.

Q19  [2M · NAT]  Bandwidth-delay product (bits) for a 1 Mbps link with 10 ms propagation delay?

Q20  [2M · MCQ]  NAT maps:  (a) names to IPs  (b) private to public addresses  (c) ports to protocols  (d) IPs to MACs.

## 7.6  Answer key and full solutions

COMMON MISTAKE / TRAP — Spoilers ahead Only read after attempting every question.

### 7.6.1  Part A

Q1.  /26 ⇒ 6 host bits ⇒ 2⁶ − 2 = 62.  Answer: 62.

Q2.  TCP is connection-oriented and reliable.  Answer: (b).

Q3.  Routing is the network layer.  Answer: (b).

Q4.  A /24 mask has 24 one-bits.  Answer: 24.

Q5.  Ethernet uses CSMA/CD.  Answer: (b).

Q6.  TCP is reliable, connection-oriented, with flow control; it is NOT connectionless. Answer: (a),(b),(c).

Q7.  1000 / 10⁶ = 10⁻³ s = 1 ms.  Answer: 1.

Q8.  DNS uses UDP port 53.  Answer: (b).

### 7.6.2  Part B

Q9.  /30 ⇒ 2² − 2 = 2 usable hosts.  Answer: 2.

Q10.  Distance-vector = Bellman–Ford.  Answer: (b).

Q11.  /28 ⇒ 2⁴ = 16 addresses.  Answer: 16.

Q12.  Link-state = Dijkstra.  Answer: (b).

Q13.  1 → 2 → 4 → 8 after 3 RTTs.  Answer: 8.

Q14.  10.0.0.0/8, 192.168.0.0/16, 172.16.0.0/12 are private; 8.8.8.0/24 is public. Answer: (a),(b),(d).

Q15.  800 / 8 = 100.  Answer: 100.

Q16.  Flow control protects the receiver (its buffer).  Answer: (b).

Q17.  200000 / (2×10⁸) = 10⁻³ s = 1 ms.  Answer: 1.

Q18.  404 = client error (resource not found).  Answer: (c).

Q19.  10⁶ × 0.01 = 10,000 bits.  Answer: 10000.

Q20.  NAT maps private to public IP addresses.  Answer: (b).

## 7.7  Document CS-CN — coverage checklist & verification

| Done | Official sub-topic | Covered in |
| --- | --- | --- |
| ☐ | Principles of layering | Ch. 1 |
| ☐ | Switching (circuit/packet/virtual) & performance metrics | Ch. 2 |
| ☐ | Data link: error detection, MAC, Ethernet | Ch. 3 |
| ☐ | Distance-vector & link-state routing | Ch. 4 |
| ☐ | IPv4: fragmentation, CIDR, NAT | Ch. 4 |
| ☐ | TCP: flow & congestion control, socket API | Ch. 5 |
| ☐ | DNS and HTTP | Ch. 6 |
| NOTE — Verification All worked examples and the 20-question answer key were computed and cross-checked. Syllabus basis: official GATE 2027 (IIT Madras), per Document 00. This completes the CS-only subjects. With SH1–SH6 and GA, the entire CS paper is covered. No PYQ is fabricated; verified PYQs with sources are in PYQ-CS. |  |  |  |