GATE 2027 PREPARATION LIBRARY

GATE CS — Computer Science & Information Technology

Computer Organization & Architecture

Instruction sets, control, memory hierarchy, I/O and pipelining

Document CS-COA

First principles → address/cache/pipeline calculations → GATE-level numericals. A heavily-tested, calculation-rich CS subject.

| Document metadata |  |
| --- | --- |
| Serves | GATE CS (Section 3: Computer Organization and Architecture) |
| Syllabus basis | Official GATE 2027 (IIT Madras), verified 12 Oct 2026 — see Document 00 |
| Scope | Instruction set & addressing modes; ALU design; control unit (hardwired/microprogrammed); memory interfacing & hierarchy (cache mapping, performance); I/O (interrupt, DMA); instruction pipelining & hazards |
| Prerequisites | CS-DL (Digital Logic) |
| Version | v1.0 — 14 Oct 2026 |

Contents

Chapter 1:  Subject Overview	4

1.1  Why COA matters for GATE CS	4

1.2  Chapter checklist	4

Chapter 2:  Instruction Set and Addressing Modes	5

2.1  ISA basics	5

2.2  Addressing modes	5

2.3  Chapter 2 summary	6

Chapter 3:  The ALU and Datapath	7

3.1  The ALU	7

3.2  The datapath and instruction cycle	7

3.3  Chapter 3 summary	7

Chapter 4:  The Control Unit	8

4.1  Hardwired vs microprogrammed	8

4.2  Chapter 4 summary	8

Chapter 5:  Memory Hierarchy and Cache	9

5.1  Locality and the hierarchy	9

5.2  Cache mapping and the address split	9

5.3  Performance: AMAT	9

5.4  Chapter 5 summary	10

Chapter 6:  I/O Interface: Interrupts and DMA	11

6.1  The three I/O methods	11

6.2  Interrupts and DMA details	11

6.3  Chapter 6 summary	11

Chapter 7:  Instruction Pipelining and Hazards	12

7.1  Pipeline speed-up	12

7.2  Hazards	12

7.3  Chapter 7 summary	13

Chapter 8:  Formula Sheet, Practice Set and Coverage	14

8.1  Formula sheet	14

8.2  A note on previous-year questions (PYQs)	14

8.3  Practice set — instructions	14

8.4  Part A — one-mark questions	14

8.5  Part B — two-mark questions	15

8.6  Answer key and full solutions	16

8.6.1  Part A	16

8.6.2  Part B	16

8.7  Document CS-COA — coverage checklist & verification	16

# Chapter 1:  Subject Overview

## 1.1  Why COA matters for GATE CS

Computer Organization is calculation-heavy and high-weight. Cache address breakdowns, AMAT, and pipeline speed-up are near-certain numericals. It builds on Digital Logic (CS-DL) and connects to Operating Systems (memory).

| Official sub-topic | Where |
| --- | --- |
| Instruction set and addressing modes | Ch. 2 |
| Design of arithmetic and logic unit (ALU) | Ch. 3 |
| Design of control unit — hardwired and microprogrammed | Ch. 4 |
| Memory interfacing and hierarchy: performance, cache mapping | Ch. 5 |
| I/O interface (interrupt and DMA) | Ch. 6 |
| Instruction pipelining, pipeline hazards | Ch. 7 |

Table. The CS COA syllabus.

INTUITION — The theme COA is "how the CPU actually runs instructions fast": fetch/decode/execute, keep data close (caches), overlap work (pipelining), and move data in/out (I/O). Most marks come from three calculations — cache address split, AMAT, and pipeline speed-up.

## 1.2  Chapter checklist

| Done | Chapter | You should be able to… |
| --- | --- | --- |
| ☐ | Ch. 2 ISA & addressing | Compute effective addresses for each addressing mode. |
| ☐ | Ch. 3 ALU & datapath | Describe the datapath and ALU operations. |
| ☐ | Ch. 4 Control unit | Contrast hardwired vs microprogrammed control. |
| ☐ | Ch. 5 Cache & memory | Split an address into tag/index/offset; compute AMAT. |
| ☐ | Ch. 6 I/O | Compare programmed, interrupt-driven and DMA I/O. |
| ☐ | Ch. 7 Pipelining | Compute speed-up; identify and resolve hazards. |
| ☐ | Ch. 8 Formulas & practice | Recall every formula and clear timed numericals. |

# Chapter 2:  Instruction Set and Addressing Modes

The instruction set (ISA) is the programmer-visible interface; addressing modes are the ways an instruction names its operands.

## 2.1  ISA basics

KEY FORMULA — Instruction-set facts An instruction has an opcode plus operands; formats vary (0/1/2/3-address machines — stack, accumulator, register). RISC: fixed-length, simple instructions, load/store architecture. CISC: variable-length, complex instructions. More addressing modes and instruction types ⇒ fewer instructions per program but more complex decoding.

## 2.2  Addressing modes

| Mode | Operand is… | Effective address (EA) |
| --- | --- | --- |
| Immediate | the constant in the instruction | (no memory access) |
| Register | in a register | (register, no memory) |
| Direct (absolute) | at the address in the instruction | EA = address field |
| Indirect | at the address stored at the given address | EA = Mem[address field] |
| Register indirect | at the address held in a register | EA = Reg[R] |
| Indexed | base + index register | EA = address + Reg[index] |
| Base + displacement | base register + offset | EA = Reg[base] + offset |
| PC-relative | relative to the program counter | EA = PC + offset |

Table. EA = effective address. Immediate/register modes avoid memory; indirect needs an EXTRA memory access.

EXAMPLE — Example 2.1 · Computing an effective address · Level 2 Indexed mode with address field = 1000 and index register = 25: EA = 1000 + 25 = 1025, and the operand is Mem[1025]. PC-relative with PC = 2000 and offset = −8: EA = 1992 (used for position-independent branches).

COMMON MISTAKE / TRAP — Addressing traps Indirect addressing needs an EXTRA memory read to fetch the address before the operand. Immediate = the value itself; direct = the value’s address — do not confuse them. PC-relative and base+displacement enable relocatable code.

## 2.3  Chapter 2 summary

KEY TAKEAWAYS — Key takeaways ISA = opcode + operands; RISC (fixed/simple) vs CISC (variable/complex). Addressing modes: immediate, register, direct, indirect, register-indirect, indexed, base+disp, PC-relative. Indirect costs an extra memory access; immediate/register avoid memory.

# Chapter 3:  The ALU and Datapath

The datapath carries data between registers through the ALU, orchestrated by the control unit (Ch. 4).

## 3.1  The ALU

DEFINITION — Arithmetic and Logic Unit The ALU performs arithmetic (add, subtract) and logic (AND, OR, XOR, shifts) on operands, selected by control/function bits. It produces a result plus status FLAGS: Zero, Carry, Sign (negative), and Overflow — used by conditional branches. Built from full adders (CS-DL) plus logic gates; subtraction uses 2’s-complement addition.

## 3.2  The datapath and instruction cycle

KEY FORMULA — Fetch–decode–execute Registers: PC (next instruction address), IR (current instruction), MAR/MDR (memory address/data), general-purpose registers. Cycle: FETCH (IR ← Mem[PC]; PC ← PC+1) → DECODE (interpret opcode) → EXECUTE (ALU op / memory access / branch). Each micro-operation is a register transfer (e.g. MAR ← PC) driven by control signals.

COMMON MISTAKE / TRAP — ALU/datapath traps The flags (Z, C, S, V) are set by the ALU and read by branch instructions — know which operation sets which. Subtraction and comparison share the adder via 2’s complement. The datapath is combinational+registers; the CONTROL unit (next chapter) decides which operation happens when.

## 3.3  Chapter 3 summary

KEY TAKEAWAYS — Key takeaways ALU does arithmetic/logic and sets flags Z, C, S, V. Instruction cycle: fetch → decode → execute, as register transfers. Datapath = registers + ALU + buses; control sequences it.

# Chapter 4:  The Control Unit

The control unit generates the signals that drive the datapath each cycle. Two design philosophies: hardwired and microprogrammed.

## 4.1  Hardwired vs microprogrammed

| Aspect | Hardwired | Microprogrammed |
| --- | --- | --- |
| How signals are generated | Fixed logic (gates, FSM) | Microinstructions in a control memory |
| Speed | Faster | Slower (extra memory access) |
| Flexibility | Hard to modify (rewire) | Easy to modify/extend (rewrite microcode) |
| Complexity handling | Best for simple/RISC control | Best for complex/CISC instruction sets |

Table. The classic trade-off: hardwired is fast but rigid; microprogrammed is flexible but slower.

KEY FORMULA — Microprogramming terms A microinstruction holds the control signals for one step; a sequence (microprogram/microroutine) implements one machine instruction. Horizontal microcode: wide, one bit per control signal (fast, more memory). Vertical microcode: encoded fields (compact, needs decoding). The control memory stores the microprogram; a microsequencer picks the next microinstruction.

COMMON MISTAKE / TRAP — Control-unit traps Hardwired = speed, microprogrammed = flexibility — match each to its use (RISC vs CISC). Horizontal microcode is wider/faster; vertical is narrower but needs decoding — a common MSQ.

## 4.2  Chapter 4 summary

KEY TAKEAWAYS — Key takeaways Control unit generates datapath signals each cycle. Hardwired: fast, rigid, suits RISC; microprogrammed: flexible, slower, suits CISC. Horizontal (wide, fast) vs vertical (encoded, compact) microcode.

# Chapter 5:  Memory Hierarchy and Cache

The memory hierarchy (registers → cache → RAM → disk) exploits LOCALITY to give the illusion of large, fast memory. Cache calculations are prime GATE numericals.

## 5.1  Locality and the hierarchy

DEFINITION — Why caches work Temporal locality: recently-used data is likely used again soon. Spatial locality: nearby data is likely used soon. Each level up is smaller, faster and costlier per bit; a cache holds recently/nearby-used blocks of main memory.

## 5.2  Cache mapping and the address split

| Mapping | A block can go… | Index selects |
| --- | --- | --- |
| Direct-mapped | exactly ONE line: (block no.) mod (#lines) | the line |
| Fully associative | ANY line | (no index; search all tags) |
| k-way set-associative | any line within ONE set: (block no.) mod (#sets) | the set |

Table. An address splits into TAG | INDEX | OFFSET. Offset = log₂(block size); Index = log₂(#lines or #sets); Tag = remaining bits.

EXAMPLE — Example 5.1 · Splitting an address · Level 3 32-bit address, 16 KB direct-mapped cache, 32-byte blocks. Offset = log₂32 = 5 bits. #lines = 16 KB / 32 B = 512 = 2⁹ ⇒ Index = 9 bits. Tag = 32 − 5 − 9 = 18 bits. For a 4-way set-associative cache of the same size, #sets = 512/4 = 128 = 2⁷ ⇒ Index = 7, Tag = 32 − 5 − 7 = 20 bits.

## 5.3  Performance: AMAT

| AMAT = Hit time + Miss rate × Miss penalty | (5.1) |
| --- | --- |
| EXAMPLE — Example 5.2 · Average memory access time · Level 3 Hit time = 1 cycle, miss rate = 5%, miss penalty = 100 cycles. AMAT = 1 + 0.05 × 100 = 6 cycles. Hit ratio = 1 − miss rate = 0.95. For multi-level caches, AMAT = Hit_L1 + MissRate_L1 × (Hit_L2 + MissRate_L2 × Penalty). |  |  |
| KEY FORMULA — Write policies Write-through: write to cache AND memory (simple, consistent, more traffic). Write-back: write to cache only, flush on eviction (less traffic, needs a dirty bit). Write-allocate (fetch block on a write miss) vs no-write-allocate. |  |  |
| COMMON MISTAKE / TRAP — Cache traps Offset = log₂(block size in BYTES); index = log₂(number of lines for direct, number of SETS for set-associative). Set-associative reduces conflict misses vs direct-mapped; fully associative has none (only capacity/compulsory). Increasing block size helps spatial locality but too large raises miss penalty and can hurt. |  |  |

## 5.4  Chapter 5 summary

KEY TAKEAWAYS — Key takeaways Locality (temporal/spatial) makes caching work. Address = Tag ¦ Index ¦ Offset; offset = log₂(block), index = log₂(#lines or #sets). Direct-mapped (1 line) / set-associative (1 set) / fully associative (any line). AMAT = hit time + miss rate × miss penalty; write-through vs write-back.

QUICK REVISION — Can you do these without notes? Split an address into tag/index/offset for any mapping. Compute AMAT (single and multi-level). Contrast the three mappings and the write policies.

# Chapter 6:  I/O Interface: Interrupts and DMA

How the CPU exchanges data with devices. The three methods trade CPU involvement against complexity.

## 6.1  The three I/O methods

| Method | How | CPU cost |
| --- | --- | --- |
| Programmed I/O (polling) | CPU repeatedly checks a status flag and transfers each word | Very high — CPU busy-waits |
| Interrupt-driven | Device INTERRUPTS the CPU when ready; CPU services it then resumes | Lower — no busy-wait, but per-word overhead |
| DMA | A DMA controller transfers a whole block directly to/from memory | Lowest — CPU only sets up & is interrupted at the end |

Table. DMA frees the CPU for bulk transfers; interrupts avoid polling; programmed I/O is simplest but wastes CPU.

## 6.2  Interrupts and DMA details

KEY FORMULA — Key mechanisms Interrupts: the CPU finishes the current instruction, saves state, jumps to an interrupt service routine (ISR), then restores state. Priorities and vectored interrupts decide which ISR runs. DMA: the controller takes the bus to move a block; "cycle stealing" interleaves DMA with CPU bus use; "burst mode" transfers the whole block at once. The CPU is interrupted only when the transfer completes.

COMMON MISTAKE / TRAP — I/O traps Programmed I/O keeps the CPU 100% busy polling — fine only for slow/simple devices. DMA still interrupts the CPU ONCE (at completion); it eliminates per-word CPU work, not all involvement. DMA and the CPU contend for the bus (cycle stealing) — a frequent performance point.

## 6.3  Chapter 6 summary

KEY TAKEAWAYS — Key takeaways Programmed I/O (polling): simplest, CPU busy-waits. Interrupt-driven: device signals readiness; no busy-wait. DMA: controller moves whole blocks; CPU set-up + one completion interrupt; shares the bus (cycle stealing).

# Chapter 7:  Instruction Pipelining and Hazards

Pipelining overlaps the stages of consecutive instructions — like an assembly line — to raise throughput. Speed-up and hazard questions are GATE staples.

## 7.1  Pipeline speed-up

A classic 5-stage pipeline: IF (fetch) → ID (decode) → EX (execute) → MEM (memory) → WB (write-back). With k stages and n instructions:

| Speed-up = n × kk + (n - 1) → k as n → ∞ | (7.1) |
| --- | --- |
| EXAMPLE — Example 7.1 · Computing speed-up · Level 3 5-stage pipeline, 100 instructions. Non-pipelined ≈ n·k = 500 cycles; pipelined = k + (n−1) = 5 + 99 = 104 cycles. Speed-up = 500/104 ≈ 4.81 (approaching the ideal 5). Ideal CPI after the pipeline fills is 1. |  |  |

## 7.2  Hazards

| Hazard | Cause | Fix |
| --- | --- | --- |
| Structural | Two instructions need the same resource at once | Duplicate/segment the resource (e.g. separate I/D caches) |
| Data (RAW) | An instruction needs a result not yet written | Forwarding/bypassing; stall (bubble) if unavoidable |
| Control | A branch changes the next PC, after later instructions are fetched | Branch prediction; delayed branch; stall/flush |

Table. RAW (read-after-write) is the common data hazard; WAR/WAW arise with out-of-order execution.

KEY FORMULA — Resolving hazards Forwarding (bypassing): route an ALU result directly to a dependent instruction instead of waiting for write-back — removes many RAW stalls. Stalls (bubbles) delay the pipeline when forwarding cannot help (e.g. a load-use hazard needs one stall). Control hazards: predict branches (static/dynamic) or reorder; a mispredict flushes the wrongly-fetched instructions.

COMMON MISTAKE / TRAP — Pipeline traps Speed-up approaches the number of stages k only for large n; hazards and stalls reduce it below ideal. Forwarding removes MANY but not all data-hazard stalls (a load-use still costs one cycle). Deeper pipelines raise clock speed but increase branch-mispredict penalty.

## 7.3  Chapter 7 summary

KEY TAKEAWAYS — Key takeaways Pipelined time = k + (n−1) cycles; speed-up = nk/(k+n−1) → k. Hazards: structural (resource), data RAW (forwarding/stall), control (branch prediction). Forwarding cuts most RAW stalls; mispredicted branches flush the pipeline.

QUICK REVISION — Can you do these without notes? Compute pipeline cycles and speed-up. Classify a hazard and give its fix. Explain forwarding, the load-use stall, and branch penalties.

# Chapter 8:  Formula Sheet, Practice Set and Coverage

## 8.1  Formula sheet

| Topic | Formula / fact |
| --- | --- |
| Addressing | indexed EA = addr + Reg[index]; indirect needs an extra memory access |
| Cache split | offset = log₂(block); index = log₂(#lines or #sets); tag = rest |
| AMAT | hit time + miss rate × miss penalty |
| Hit ratio | 1 − miss rate |
| Pipeline time | k + (n − 1) cycles |
| Pipeline speed-up | nk/(k + n − 1) → k |
| Hazards | structural / data (RAW) / control |
| Control unit | hardwired (fast/rigid) vs microprogrammed (flexible/slow) |
| I/O | programmed (poll) / interrupt / DMA (block, one completion interrupt) |
| Write policy | write-through vs write-back (needs dirty bit) |

Table. One-page COA recap (consolidated in the Formula Book, M2).

## 8.2  A note on previous-year questions (PYQs)

COMMON MISTAKE / TRAP — No fabricated PYQs This library never invents a past question, year or official answer. Verified PYQs with sources are in PYQ-CS. Below is ORIGINAL GATE-style practice.

## 8.3  Practice set — instructions

Attempt timed before the solutions. Part A = 1-mark; Part B = 2-mark. GATE marking as usual.

NOTE — Answer key is separated Solutions start on a new page (Section 8.5).

## 8.4  Part A — one-mark questions

Q1  [1M · MCQ]  In IMMEDIATE addressing, the operand is:  (a) in a register  (b) the constant in the instruction  (c) at the given address  (d) at an address in a register.

Q2  [1M · NAT]  How many OFFSET bits does a 64-byte cache block need?

Q3  [1M · MCQ]  AMAT equals:  (a) hit time × miss rate  (b) hit time + miss rate × miss penalty  (c) miss penalty only  (d) 1 − miss rate.

Q4  [1M · MCQ]  DMA is used to:  (a) poll a device  (b) transfer a block directly to/from memory  (c) decode instructions  (d) predict branches.

Q5  [1M · NAT]  If the cache miss rate is 0.1, what is the hit ratio?

Q6  [1M · MSQ]  Which are pipeline HAZARDS? (a) structural  (b) data  (c) control  (d) arithmetic.

Q7  [1M · NAT]  How many INDEX bits for a direct-mapped cache with 256 lines?

Q8  [1M · MCQ]  Hardwired control, compared to microprogrammed, is:  (a) slower but flexible  (b) faster but less flexible  (c) identical  (d) always microcoded.

## 8.5  Part B — two-mark questions

Q9  [2M · NAT]  Hit time = 2 cycles, miss rate = 0.1, miss penalty = 50 cycles. Compute AMAT.

Q10  [2M · NAT]  32-bit address, 32-byte blocks, direct-mapped with 1024 lines. How many TAG bits?

Q11  [2M · NAT]  A 4-stage pipeline executes 50 instructions. How many cycles does it take (k + n − 1)?

Q12  [2M · NAT]  Speed-up of a 5-stage pipeline for 100 instructions (nk/(k+n−1), 2 decimals)?

Q13  [2M · MCQ]  A RAW data hazard is commonly resolved by:  (a) forwarding  (b) a larger cache  (c) DMA  (d) more registers only.

Q14  [2M · NAT]  A 4-way set-associative cache has 512 lines. How many SETS does it have?

Q15  [2M · MSQ]  Which I/O methods AVOID CPU busy-waiting? (a) interrupt-driven  (b) DMA  (c) programmed I/O  (d) polling.

Q16  [2M · NAT]  Indexed addressing: address field = 2000, index register = 50. What is the effective address?

Q17  [2M · MCQ]  Microprogrammed control is:  (a) faster, rigid  (b) more flexible, slower  (c) hardwired  (d) impossible for CISC.

Q18  [2M · NAT]  Two-level cache: L1 hit 1, L1 miss 0.1; L2 hit 10, L2 miss 0.2; memory 100 cycles. Compute AMAT.

Q19  [2M · NAT]  What is the ideal CPI of a pipelined processor (after the pipeline is full)?

Q20  [2M · MCQ]  Spatial locality means:  (a) recently-used data is reused  (b) nearby data is likely used soon  (c) data never repeats  (d) caches are large.

Q21  [2M · NAT]  A 32-KB cache uses 64-byte blocks. How many lines (blocks) does it have?

Q22  [2M · MCQ]  A write-back cache requires a:  (a) valid bit only  (b) dirty bit  (c) tag only  (d) parity bit.

## 8.6  Answer key and full solutions

COMMON MISTAKE / TRAP — Spoilers ahead Only read after attempting every question.

### 8.6.1  Part A

Q1.  Immediate: the operand is the constant in the instruction.  Answer: (b).

Q2.  log₂64 = 6 offset bits.  Answer: 6.

Q3.  AMAT = hit time + miss rate × miss penalty.  Answer: (b).

Q4.  DMA transfers blocks directly to/from memory.  Answer: (b).

Q5.  Hit ratio = 1 − 0.1 = 0.9.  Answer: 0.9.

Q6.  Structural, data and control are the three hazards. Answer: (a),(b),(c).

Q7.  log₂256 = 8 index bits.  Answer: 8.

Q8.  Hardwired is faster but less flexible.  Answer: (b).

### 8.6.2  Part B

Q9.  AMAT = 2 + 0.1 × 50 = 2 + 5 = 7 cycles.  Answer: 7.

Q10.  Offset = log₂32 = 5; index = log₂1024 = 10; tag = 32 − 5 − 10 = 17.  Answer: 17.

Q11.  k + (n−1) = 4 + 49 = 53 cycles.  Answer: 53.

Q12.  nk/(k+n−1) = 500/104 ≈ 4.81.  Answer: 4.81.

Q13.  Forwarding (bypassing) resolves RAW hazards.  Answer: (a).

Q14.  #sets = 512 / 4 = 128.  Answer: 128.

Q15.  Interrupt-driven and DMA avoid busy-waiting; programmed I/O/polling do not. Answer: (a),(b).

Q16.  EA = 2000 + 50 = 2050.  Answer: 2050.

Q17.  Microprogrammed = more flexible, slower.  Answer: (b).

Q18.  AMAT = 1 + 0.1 × (10 + 0.2 × 100) = 1 + 0.1 × 30 = 4 cycles.  Answer: 4.

Q19.  Ideal pipelined CPI = 1.  Answer: 1.

Q20.  Spatial locality: nearby data likely used soon.  Answer: (b).

Q21.  32 KB / 64 B = 32768/64 = 512 lines.  Answer: 512.

Q22.  Write-back needs a dirty bit.  Answer: (b).

## 8.7  Document CS-COA — coverage checklist & verification

| Done | Official sub-topic | Covered in |
| --- | --- | --- |
| ☐ | Instruction set and addressing modes | Ch. 2 |
| ☐ | Design of ALU | Ch. 3 |
| ☐ | Control unit — hardwired & microprogrammed | Ch. 4 |
| ☐ | Memory interfacing & hierarchy: performance, cache mapping | Ch. 5 |
| ☐ | I/O interface (interrupt and DMA) | Ch. 6 |
| ☐ | Instruction pipelining, pipeline hazards | Ch. 7 |
| NOTE — Verification All worked examples and the 22-question answer key were computed and cross-checked. Syllabus basis: official GATE 2027 (IIT Madras), per Document 00. No PYQ is fabricated; verified PYQs with sources are in PYQ-CS. |  |  |  |