GATE 2027 PREPARATION LIBRARY

GATE CS — Computer Science & Information Technology

Digital Logic

Number systems, Boolean algebra, minimization, and combinational & sequential circuits

Document CS-DL

First principles → truth tables & K-maps → GATE-level circuit design and arithmetic. A high-scoring, mechanical CS subject.

| Document metadata |  |
| --- | --- |
| Serves | GATE CS (Section 2: Digital Logic) |
| Syllabus basis | Official GATE 2027 (IIT Madras), verified 12 Oct 2026 — see Document 00 |
| Scope | Boolean algebra & minimization (algebraic, Karnaugh map, tabular); combinational & sequential circuit design; number representation & arithmetic (fixed and floating point) |
| Version | v1.0 — 14 Oct 2026 |

Contents

Chapter 1:  Subject Overview	4

1.1  Why Digital Logic matters for GATE CS	4

1.2  How it is tested	4

1.3  Chapter checklist	4

Chapter 2:  Number Systems and Representation	5

2.1  Bases and conversion	5

2.2  Signed-number representations (n bits)	5

2.3  Chapter 2 summary	5

Chapter 3:  Fixed and Floating Point Arithmetic	6

3.1  2’s-complement addition and overflow	6

3.2  Floating point: IEEE 754	6

3.3  Chapter 3 summary	6

Chapter 4:  Boolean Algebra	8

4.1  The laws	8

4.2  Canonical forms	8

4.3  Chapter 4 summary	8

Chapter 5:  Minimization: Karnaugh Maps and the Tabular Method	10

5.1  Karnaugh maps	10

5.2  Prime implicants	10

5.3  The tabular (Quine–McCluskey) method	10

5.4  Chapter 5 summary	11

Chapter 6:  Combinational Circuits	12

6.1  Adders	12

6.2  Multiplexers, decoders, encoders	12

6.3  Chapter 6 summary	12

Chapter 7:  Sequential Circuits	14

7.1  Latches and flip-flops	14

7.2  Counters and registers	14

7.3  Chapter 7 summary	14

Chapter 8:  Formula Sheet, Practice Set and Coverage	16

8.1  Formula sheet	16

8.2  A note on previous-year questions (PYQs)	16

8.3  Practice set — instructions	16

8.4  Part A — one-mark questions	16

8.5  Part B — two-mark questions	17

8.6  Answer key and full solutions	18

8.6.1  Part A	18

8.6.2  Part B	18

8.7  Document CS-DL — coverage checklist & verification	18

# Chapter 1:  Subject Overview

## 1.1  Why Digital Logic matters for GATE CS

Digital Logic is one of the most mechanical and scorable CS subjects: once you can minimise a Boolean function, analyse a flip-flop, and do 2’s-complement/IEEE arithmetic, the questions become routine. It also feeds directly into Computer Organization (CS-COA).

| Official sub-topic | Where |
| --- | --- |
| Number representation & arithmetic (fixed & floating point) | Ch. 2–3 |
| Boolean algebra | Ch. 4 |
| Minimization: algebraic, Karnaugh map, tabular | Ch. 5 |
| Combinational circuit design | Ch. 6 |
| Sequential circuit design | Ch. 7 |

Table. The CS Digital-Logic syllabus.

INTUITION — The theme Everything reduces to 0s and 1s. Boolean algebra manipulates them, K-maps minimise the logic, combinational circuits compute functions of the present inputs, and sequential circuits add memory (state).

## 1.2  How it is tested

Expect NAT 2’s-complement/IEEE conversions and ranges, K-map minimization (count literals/terms), flip-flop next-state, and MUX/decoder implementations. Fast marks with practice.

## 1.3  Chapter checklist

| Done | Chapter | You should be able to… |
| --- | --- | --- |
| ☐ | Ch. 2 Number systems | Convert bases; use sign-magnitude, 1’s/2’s complement and ranges. |
| ☐ | Ch. 3 Arithmetic | Add/subtract in 2’s complement; read IEEE-754 fields; detect overflow. |
| ☐ | Ch. 4 Boolean algebra | Apply the laws, De Morgan, SOP/POS, minterms/maxterms. |
| ☐ | Ch. 5 Minimization | Minimise with K-maps (and the tabular method); use don’t-cares. |
| ☐ | Ch. 6 Combinational | Design adders, MUX, decoders, encoders, comparators. |
| ☐ | Ch. 7 Sequential | Analyse latches/flip-flops; design counters and registers. |
| ☐ | Ch. 8 Formulas & practice | Recall the rules and clear timed practice. |

# Chapter 2:  Number Systems and Representation

Bases and signed-number encodings — the foundation of all computer arithmetic.

## 2.1  Bases and conversion

KEY FORMULA — Base facts Binary (base 2), octal (base 8), hexadecimal (base 16). Group binary in 3s for octal, 4s for hex. A value in base b: Σ dᵢ·bⁱ. e.g. 1011₂ = 8+0+2+1 = 11₁₀. n bits represent 2ⁿ distinct values (unsigned range 0 … 2ⁿ−1).

## 2.2  Signed-number representations (n bits)

| Representation | Negate by | Range | Zeros |
| --- | --- | --- | --- |
| Sign-magnitude | flip the sign bit | −(2ⁿ⁻¹−1) … +(2ⁿ⁻¹−1) | two (+0, −0) |
| 1’s complement | invert all bits | −(2ⁿ⁻¹−1) … +(2ⁿ⁻¹−1) | two (+0, −0) |
| 2’s complement | invert all bits, then add 1 | −2ⁿ⁻¹ … +(2ⁿ⁻¹−1) | one (unique 0) |

Table. 2’s complement is used in practice: a unique zero, and addition/subtraction use the same hardware.

EXAMPLE — Example 2.1 · 2’s complement · Level 2 Represent −5 in 8-bit 2’s complement. +5 = 00000101. Invert → 11111010; add 1 → 11111011. So −5 = 11111011. 8-bit 2’s-complement range is −2⁷ … 2⁷−1 = −128 … +127.

COMMON MISTAKE / TRAP — Representation traps 2’s-complement range is ASYMMETRIC: one more negative value (−128) than positive (+127) for 8 bits. To negate in 2’s complement: invert ALL bits then add 1 (not just flip the sign bit). Sign-magnitude and 1’s complement have TWO zeros; 2’s complement has one.

## 2.3  Chapter 2 summary

KEY TAKEAWAYS — Key takeaways n bits → 2ⁿ values; unsigned 0…2ⁿ−1. 2’s complement: negate = invert + 1; range −2ⁿ⁻¹ … 2ⁿ⁻¹−1; unique zero. Sign-magnitude / 1’s complement: symmetric range, two zeros.

# Chapter 3:  Fixed and Floating Point Arithmetic

How computers add signed integers and represent real numbers (IEEE 754).

## 3.1  2’s-complement addition and overflow

KEY FORMULA — Addition rules Add in 2’s complement by ordinary binary addition and DISCARD any carry out of the sign bit. Subtraction A − B = A + (2’s complement of B). OVERFLOW occurs when two same-sign operands produce a different-sign result (e.g. positive + positive = negative). Carry-out alone is NOT overflow in 2’s complement.

EXAMPLE — Example 3.1 · Detecting overflow · Level 3 4-bit 2’s complement: 0101 (+5) + 0100 (+4) = 1001 = −7. Two positives gave a negative ⇒ OVERFLOW (the true answer +9 exceeds the +7 maximum).

## 3.2  Floating point: IEEE 754

| Format | Sign | Exponent (bias) | Mantissa | Total |
| --- | --- | --- | --- | --- |
| Single (float) | 1 | 8 (bias 127) | 23 | 32 bits |
| Double | 1 | 11 (bias 1023) | 52 | 64 bits |

Table. Value = (−1)^sign × 1.mantissa × 2^(exponent − bias). The leading 1 is implicit (normalised).

KEY FORMULA — Reading a float Stored exponent is BIASED: actual exponent = stored − bias (127 for single). Normalised numbers assume an implicit leading 1 (so the mantissa gives 24 bits of precision from 23 stored). Special patterns: exponent all 0s = zero/denormal; all 1s = ∞ (mantissa 0) or NaN (mantissa ≠ 0).

COMMON MISTAKE / TRAP — Arithmetic traps Overflow in 2’s complement is a SIGN mismatch of same-sign operands — not the carry-out bit. The IEEE exponent is biased; forgetting to subtract the bias is the classic error. Floating point is not exact (e.g. 0.1 is not representable); comparisons need a tolerance.

## 3.3  Chapter 3 summary

KEY TAKEAWAYS — Key takeaways 2’s-complement add/subtract with the same hardware; overflow = same-sign operands, opposite-sign result. IEEE 754 single = 1 + 8(bias 127) + 23; value = (−1)ˢ·1.m·2^(e−bias). Exponent is biased; implicit leading 1; special exponent fields for 0/∞/NaN.

QUICK REVISION — Can you do these without notes? Add/subtract 2’s-complement numbers and detect overflow. Decode an IEEE-754 single-precision value. State the bias and special encodings.

# Chapter 4:  Boolean Algebra

The algebra of 0/1 and the gates AND (·), OR (+), NOT (′). Mastering the laws makes minimization fast.

## 4.1  The laws

KEY FORMULA — Core identities Identity: A+0=A, A·1=A. Null: A+1=1, A·0=0. Idempotent: A+A=A, A·A=A. Complement: A+A′=1, A·A′=0. Involution: (A′)′=A. Commutative, Associative, Distributive (A·(B+C)=A·B+A·C and the dual A+B·C=(A+B)(A+C)). Absorption: A+A·B=A, A·(A+B)=A.

KEY FORMULA — De Morgan and duality De Morgan: (A+B)′ = A′·B′ and (A·B)′ = A′+B′. Duality: swap (· ↔ +) and (0 ↔ 1) to get the dual of any identity — both hold.

## 4.2  Canonical forms

DEFINITION — SOP and POS Sum of Products (SOP): OR of AND terms; the canonical SOP is a sum of MINTERMS (one per row where the function is 1). Product of Sums (POS): AND of OR terms; the canonical POS is a product of MAXTERMS (one per row where the function is 0). A minterm mᵢ and maxterm Mᵢ for the same row are complements: Mᵢ = mᵢ′.

EXAMPLE — Example 4.1 · Minterms · Level 2 For f(A,B) that is 1 on rows AB = 01 and 10: canonical SOP = A′B + AB′ (= A ⊕ B, the XOR). Its complement (rows 00, 11) gives the POS from the maxterms.

COMMON MISTAKE / TRAP — Boolean traps De Morgan flips the operator AND complements each term — do not forget to change · ↔ +. XOR A⊕B = A′B + AB′ (1 when inputs differ); XNOR is its complement (1 when inputs agree). Duality (swap ·/+ and 0/1) is not the same as complementation (which also complements the variables).

## 4.3  Chapter 4 summary

KEY TAKEAWAYS — Key takeaways Know identity/null/idempotent/complement/absorption laws and duality. De Morgan: (A+B)′=A′B′, (AB)′=A′+B′. SOP = sum of minterms (f=1 rows); POS = product of maxterms (f=0 rows); XOR = A′B+AB′.

# Chapter 5:  Minimization: Karnaugh Maps and the Tabular Method

Minimization turns a truth table into the simplest Boolean expression (fewest gates/literals). The K-map is the visual tool; Quine–McCluskey is its systematic cousin.

## 5.1  Karnaugh maps

KEY FORMULA — K-map rules Cells are arranged in GRAY-CODE order so adjacent cells differ in one variable. Group the 1s into rectangles of size 1, 2, 4, 8, … (powers of 2), as LARGE as possible; wrap-around edges are adjacent. A group of 2ᵏ cells eliminates k variables. Cover every 1 with the fewest, largest groups. DON’T-CARE (X) cells may be included in a group if they help enlarge it.

EXAMPLE — Example 5.1 · A 3-variable K-map · Level 3 f(A,B,C) = Σm(1,3,5,7). Place 1s and group (BC columns in Gray-code order 00, 01, 11, 10): A \ BC; 00; 01; 11; 10. 0; 0; 1; 1; 0. 1; 0; 1; 1; 0. The two middle columns (BC = 01 and 11) are all 1 — a group of 4 covering every C = 1 cell. So f = C (B and A are eliminated).

## 5.2  Prime implicants

DEFINITION — Prime & essential prime implicants A PRIME IMPLICANT is a group that cannot be enlarged. An ESSENTIAL prime implicant covers at least one 1 that no other prime implicant covers — it MUST be in the minimal cover. Minimal SOP = all essential PIs plus the fewest extra PIs needed to cover the rest.

## 5.3  The tabular (Quine–McCluskey) method

KEY FORMULA — Quine–McCluskey Systematic and computer-friendly (K-maps get unwieldy beyond 4–5 variables). Step 1: group minterms by the number of 1s; combine pairs differing in ONE bit (mark the differing position with "–") to form implicants; repeat. Step 2: build a prime-implicant chart and pick essential PIs, then cover the rest minimally.

COMMON MISTAKE / TRAP — Minimization traps Groups must be rectangles of size a POWER OF 2; a group of 3 or 6 is illegal. Bigger groups are better (fewer literals) — always extend to the largest legal rectangle, using wrap-around and don’t-cares. A minimal SOP need not be unique; different equally-small covers can exist.

## 5.4  Chapter 5 summary

KEY TAKEAWAYS — Key takeaways K-map: Gray-code adjacency; group 1s in power-of-2 rectangles, as large as possible, wrap-around; 2ᵏ group drops k variables. Essential prime implicants must appear; include don’t-cares to enlarge groups. Quine–McCluskey is the systematic method for many variables.

QUICK REVISION — Can you do these without notes? Fill and minimise a 3- or 4-variable K-map. Identify prime and essential prime implicants. Outline the Quine–McCluskey steps.

# Chapter 6:  Combinational Circuits

Combinational circuits compute outputs purely from the present inputs (no memory). Know the standard building blocks and their functions.

## 6.1  Adders

KEY FORMULA — Half and full adders Half adder: Sum = A ⊕ B, Carry = A·B. Full adder (with carry-in): Sum = A ⊕ B ⊕ Cin, Cout = A·B + Cin·(A ⊕ B). Ripple-carry adder chains n full adders; the carry propagates, so delay grows with n (carry-lookahead speeds this up).

## 6.2  Multiplexers, decoders, encoders

| Block | Function | Size |
| --- | --- | --- |
| Multiplexer (MUX) | Selects ONE of 2ⁿ data inputs using n select lines | 2ⁿ : 1 |
| Demultiplexer | Routes one input to one of 2ⁿ outputs | 1 : 2ⁿ |
| Decoder | n inputs → one-hot activation of one of 2ⁿ outputs | n : 2ⁿ |
| Encoder | 2ⁿ (one-hot) inputs → n-bit code | 2ⁿ : n |
| Comparator | Outputs A<B, A=B, A>B | — |

Table. A 2ⁿ:1 MUX can implement ANY Boolean function of n select variables — it is functionally complete.

INTUITION — MUX as a universal function block Feed the input variables to the select lines and the desired truth-table outputs (0, 1, or a remaining variable) to the data lines — a 2ⁿ:1 MUX then realises any n-variable function. Decoders + OR gates similarly implement any SOP.

COMMON MISTAKE / TRAP — Combinational traps Carry propagation makes a ripple adder’s delay O(n); carry-lookahead trades gates for speed. A decoder gives a ONE-HOT output (exactly one line high); an encoder reverses this (priority encoders resolve multiple highs). Combinational circuits have NO memory — the output depends only on the current inputs.

## 6.3  Chapter 6 summary

KEY TAKEAWAYS — Key takeaways Half adder Sum=A⊕B, Carry=AB; full adder Sum=A⊕B⊕Cin, Cout=AB+Cin(A⊕B). MUX 2ⁿ:1 selects one input (and is universal); decoder n:2ⁿ one-hot; encoder 2ⁿ:n. No memory — outputs are a pure function of current inputs.

# Chapter 7:  Sequential Circuits

Sequential circuits have MEMORY: the output depends on the inputs AND the stored state. Flip-flops are the storage elements.

## 7.1  Latches and flip-flops

| Element | Characteristic equation (Q_next) | Behaviour |
| --- | --- | --- |
| SR latch | Q⁺ = S + R′Q (S·R = 0) | S sets, R resets; S=R=1 is invalid |
| D flip-flop | Q⁺ = D | Stores the input on the clock edge |
| JK flip-flop | Q⁺ = J·Q′ + K′·Q | J=K=1 ⇒ TOGGLE; otherwise set/reset/hold |
| T flip-flop | Q⁺ = T ⊕ Q | T=1 toggles, T=0 holds |

Table. A latch is level-triggered; a flip-flop is edge-triggered (changes only on a clock edge).

EXAMPLE — Example 7.1 · JK next state · Level 2 A JK flip-flop with current Q = 1 and inputs J = 0, K = 1: Q⁺ = J·Q′ + K′·Q = 0·0 + 0·1 = 0 (it resets). With J = K = 1 it would toggle to 0.

## 7.2  Counters and registers

KEY FORMULA — Sequential building blocks Counters cycle through states: a mod-N counter needs ⌈log₂N⌉ flip-flops. ASYNCHRONOUS (ripple) counters clock each stage from the previous output (slower); SYNCHRONOUS counters clock all flip-flops together (faster, no ripple). Registers store/shift bits; a shift register moves data one position per clock (SISO/SIPO/PISO/PIPO). Timing: data must be stable for the SETUP time before and the HOLD time after the clock edge; max clock frequency is bounded by the longest register-to-register path.

COMMON MISTAKE / TRAP — Sequential traps A mod-N counter needs ⌈log₂N⌉ flip-flops (e.g. mod-10 needs 4). Asynchronous counters accumulate propagation delay (ripple); synchronous ones do not. Setup/hold violations cause metastability — timing is as important as logic. A latch is level-sensitive; a flip-flop is edge-triggered — do not conflate them.

## 7.3  Chapter 7 summary

KEY TAKEAWAYS — Key takeaways Flip-flops store state: D (Q⁺=D), JK (J=K=1 toggles), T (Q⁺=T⊕Q), SR (set/reset). Mod-N counter needs ⌈log₂N⌉ flip-flops; synchronous beats ripple on speed. Setup/hold timing bounds the clock frequency; latch (level) vs flip-flop (edge).

QUICK REVISION — Can you do these without notes? Compute the next state of SR/D/JK/T flip-flops. Size a mod-N counter and contrast ripple vs synchronous. Explain setup/hold time and metastability.

# Chapter 8:  Formula Sheet, Practice Set and Coverage

## 8.1  Formula sheet

| Topic | Fact |
| --- | --- |
| 2’s complement | negate = invert + 1; range −2ⁿ⁻¹ … 2ⁿ⁻¹−1; unique zero |
| Overflow | same-sign operands → opposite-sign result |
| IEEE 754 single | 1 + 8(bias 127) + 23; value (−1)ˢ·1.m·2^(e−127) |
| De Morgan | (A+B)′=A′B′; (AB)′=A′+B′ |
| XOR / XNOR | A⊕B = A′B+AB′; XNOR complement |
| Half/full adder | S=A⊕B, C=AB; full S=A⊕B⊕Cin, Cout=AB+Cin(A⊕B) |
| MUX/decoder | 2ⁿ:1 MUX (universal); decoder n:2ⁿ one-hot |
| Flip-flops | D: Q⁺=D; JK: Q⁺=JQ′+K′Q; T: Q⁺=T⊕Q |
| Counter | mod-N needs ⌈log₂N⌉ flip-flops |
| K-map | group 2ᵏ cells ⇒ eliminates k variables |

Table. One-page Digital-Logic recap (consolidated in the Formula Book, M2).

## 8.2  A note on previous-year questions (PYQs)

COMMON MISTAKE / TRAP — No fabricated PYQs This library never invents a past question, year or official answer. Verified PYQs with sources are in PYQ-CS. Below is ORIGINAL GATE-style practice.

## 8.3  Practice set — instructions

Attempt timed before the solutions. Part A = 1-mark; Part B = 2-mark. GATE marking as usual.

NOTE — Answer key is separated Solutions start on a new page (Section 8.5).

## 8.4  Part A — one-mark questions

Q1  [1M · NAT]  What is the most negative value representable in 8-bit 2’s complement?

Q2  [1M · NAT]  Convert 1011₂ to decimal.

Q3  [1M · MCQ]  A half adder’s SUM output is:  (a) A·B  (b) A+B  (c) A⊕B  (d) (A+B)′.

Q4  [1M · NAT]  What is the exponent bias in IEEE-754 single precision?

Q5  [1M · MCQ]  By De Morgan, (A + B)′ equals:  (a) A′+B′  (b) A′·B′  (c) A·B  (d) A+B.

Q6  [1M · MSQ]  Which representations have TWO zeros? (a) sign-magnitude  (b) 1’s complement  (c) 2’s complement  (d) unsigned.

Q7  [1M · NAT]  How many flip-flops does a mod-10 counter need?

Q8  [1M · MCQ]  A D flip-flop’s next state Q⁺ is:  (a) D′  (b) D  (c) D⊕Q  (d) Q.

## 8.5  Part B — two-mark questions

Q9  [2M · NAT]  How many distinct values can 8-bit 2’s complement represent?

Q10  [2M · NAT]  f(A,B,C) = Σm(1,3,5,7) minimises to how many LITERALS?

Q11  [2M · MCQ]  Overflow in 2’s-complement addition occurs when:  (a) there is a carry-out  (b) two same-sign operands give an opposite-sign result  (c) the result is zero  (d) inputs differ in sign.

Q12  [2M · NAT]  How many SELECT lines does an 8:1 multiplexer have?

Q13  [2M · NAT]  A JK flip-flop has Q = 1, J = 0, K = 1. What is Q⁺?

Q14  [2M · MSQ]  Which are COMBINATIONAL circuits? (a) multiplexer  (b) decoder  (c) JK flip-flop  (d) full adder.

Q15  [2M · NAT]  How many outputs does a 3-to-8 decoder have?

Q16  [2M · NAT]  In a 4-variable K-map, a group of 8 cells eliminates how many variables?

Q17  [2M · NAT]  A T flip-flop has Q = 0 and T = 1. What is Q⁺?

Q18  [2M · MCQ]  The Quine–McCluskey method is used for:  (a) base conversion  (b) Boolean minimization  (c) flip-flop design  (d) counting.

Q19  [2M · NAT]  A group of 4 cells in a K-map eliminates how many variables?

Q20  [2M · MCQ]  A 2ⁿ:1 multiplexer can implement:  (a) only AND  (b) any Boolean function of n select variables  (c) only XOR  (d) nothing without memory.

Q21  [2M · NAT]  How many flip-flops are needed for a mod-16 counter?

Q22  [2M · MCQ]  A disadvantage of an asynchronous (ripple) counter is:  (a) too few states  (b) accumulated propagation delay  (c) no reset  (d) it needs no clock.

## 8.6  Answer key and full solutions

COMMON MISTAKE / TRAP — Spoilers ahead Only read after attempting every question.

### 8.6.1  Part A

Q1.  −2⁷ = −128.  Answer: −128.

Q2.  1011₂ = 8+2+1 = 11.  Answer: 11.

Q3.  Half-adder Sum = A⊕B.  Answer: (c).

Q4.  Single-precision bias = 127.  Answer: 127.

Q5.  (A+B)′ = A′·B′.  Answer: (b).

Q6.  Sign-magnitude and 1’s complement have +0 and −0. Answer: (a),(b).

Q7.  ⌈log₂10⌉ = 4.  Answer: 4.

Q8.  Q⁺ = D.  Answer: (b).

### 8.6.2  Part B

Q9.  2⁸ = 256.  Answer: 256.

Q10.  All odd minterms ⇒ f = C ⇒ 1 literal.  Answer: 1.

Q11.  Same-sign operands producing an opposite-sign result.  Answer: (b).

Q12.  8:1 MUX needs log₂8 = 3 select lines.  Answer: 3.

Q13.  Q⁺ = J·Q′ + K′·Q = 0·0 + 0·1 = 0.  Answer: 0.

Q14.  MUX, decoder, full adder are combinational; a JK flip-flop has memory. Answer: (a),(b),(d).

Q15.  3-to-8 decoder has 2³ = 8 outputs.  Answer: 8.

Q16.  A group of 8 = 2³ cells eliminates 3 variables.  Answer: 3.

Q17.  Q⁺ = T⊕Q = 1⊕0 = 1.  Answer: 1.

Q18.  Quine–McCluskey minimises Boolean functions.  Answer: (b).

Q19.  A group of 4 = 2² cells eliminates 2 variables.  Answer: 2.

Q20.  A 2ⁿ:1 MUX is functionally complete for n select variables.  Answer: (b).

Q21.  ⌈log₂16⌉ = 4.  Answer: 4.

Q22.  Ripple counters accumulate propagation delay.  Answer: (b).

## 8.7  Document CS-DL — coverage checklist & verification

| Done | Official sub-topic | Covered in |
| --- | --- | --- |
| ☐ | Number representation & arithmetic (fixed & floating point) | Ch. 2–3 |
| ☐ | Boolean algebra | Ch. 4 |
| ☐ | Minimization — algebraic, Karnaugh map, tabular | Ch. 5 |
| ☐ | Combinational circuit design | Ch. 6 |
| ☐ | Sequential circuit design | Ch. 7 |
| NOTE — Verification All worked examples and the 22-question answer key were computed and cross-checked. Syllabus basis: official GATE 2027 (IIT Madras), per Document 00. No PYQ is fabricated; verified PYQs with sources are in PYQ-CS. |  |  |  |